Начало формы

Конец формы

|  | WARNUNG |
| --- | --- |
| Gefahr des Verlusts der Text-Bild-Zuordnung beim Ausdruck. - Aus technischen Gründen kann derzeit nicht ausgeschlossen werden, dass trotz einwandfreier Darstellung auf dem Bildschirm beim Ausdruck der Zusammenhang von Text und zugehörigem Bild verloren geht. - Wird der Fehler nicht bemerkt, besteht die Gefahr, daß Arbeiten nicht korrekt durchgeführt werden und es dadurch zu Personen- und Sachschäden kommt. → Vergleichen Sie deshalb nach dem Druckvorgang unbedingt die Ausdrucke mit der Bildschirmdarstellung. Korrigieren Sie die Ausdrucke im Fehlerfall so, daß der Inhalt der Bildschirmdarstellung zutreffend wiedergegeben wird. |  |
|  | WARNING |
| Danger of losing text-figure correspondence when printing out. - For technical reasons, it can currently not be ruled out that the correspondence between text and associated figure can be lost when it is printed out, even though the display on the screen is correct. - If the fault is not noticed, there is the danger that work will not be performed correctly which can cause injury and damage. → Therefore, always compare printouts with the screen display after printing them out. Correct the printouts if they are faulty so that the content of the screen display matches that on the printouts. |  |
|  | AVERTISSEMENT |
| Risque de perte de la correspondance texte-illustration lors de l'impression papier. - Pour des raisons techniques, il ne peut pas être exclu à l'heure actuelle que, même en cas d'affichage correct à l'écran, la correspondance entre le texte et l'illustration puisse être erronée sur l'impression papier. - Si l'erreur passe inaperçue, on court le risque d'exécuter certaines opérations de manière incorrecte pouvant conduire à des dommages corporels et matériels. → Comparez impérativement pour cette raison les impressions papier avec l'affichage à l'écran. Corrigez-les en cas d'erreur de manière à reproduire correctement l'affichage à l'écran. |  |
|  | ATTENZIONE |
| Pericolo di perdita dell'abbinamento testo-immagine nella stampa. - Per motivi tecnici non è escluso che, nonostante la rappresentazione sullo schermo risulti perfetta, durante la stampa venga persa la corrispondenza fra testo e relativa immagine. - Se tale difetto non viene rilevato vi è il pericolo che i lavori non siano eseguiti correttamente; ciò può avere come conseguenza lesioni e danneggiamenti. → Per questo motivo, dopo la stampa è assolutamente necessario confrontare le stampe con la rappresentazione sullo schermo. In caso di errore si prega di correggere le stampe, così da garantire la riproduzione esatta della rappresentazione sullo schermo. |  |
|  | ADVERTENCIA |
| Al imprimir se corre el riesgo de perder la correcta asignación de las figuras al texto. - Por motivos técnicos, por el momento no se puede descartar que, aun siendo correcta la imagen que aparezca en la pantalla, al imprimir se pierda la correlación entre el texto y la figura correspondiente. - Si este error pasara desapercibido, existe el riesgo de que los trabajos no se ejecuten de forma correcta y, por tanto, se produzcan daños personales y materiales. → Por ello le rogamos que tras el proceso de impresión compare siempre las copias impresas con la imagen de la pantalla. En caso de error, corrija las copias impresas de forma que se correspondan con el contenido de la imagen de la pantalla. |  |

- 48 92 19 Removing and installing radiator for power steering - as of MY 2003

- Preliminary work

- Removing radiator for power steering

- Installing radiator for power steering

- Subsequent work

top of page

- Preliminary work

- Preliminary work

| 1. | Raise the vehicle → Lifting the vehicle [9PAAD1 9PAAD7 9PAAE1 9PAAE7 9PAAF1]→ Lifting the vehicle [9PAAJ1]. |
| --- | --- |

| 2. | Remove engine guard → 108019 Removing and installing engine guard - section on "Removing" |
| --- | --- |

| 3. | Remove front apron → 631519 Removing and installing front apron - section on "Removing" |
| --- | --- |

top of page

- Removing radiator for power steering

- Removing radiator for power steering

The -cooler for power steering system- is located at the -radiator module (radiator, condenser, ATF cooler, cooler for power steering system)- , behind the front apron.

Servo oil cooler

|  | ATTENTION |
| --- | --- |

Increased risk of accident from Pentosin CHF 11 S getting onto the surrounding area (e.g. onto the coolant hoses) when the oil coolant hoses are loosened (opened).

- Possible danger of injury or damage.

| → | Collect any emerging Pentosin. |
| --- | --- |
| → | Wear protective goggles when carrying out overhead work on lines with Pentosin. |
| → | Wear protective gloves. |
| → | If coolant hoses come into contact with Pentosin, thoroughly clean them with water IMMEDIATELY. |
| → | Replace visibly swollen coolant hoses. |

| 1. | Remove lateral air guides and the air guide at the top -arrows- . |
| --- | --- |

|  | Note |
| --- | --- |

- For the Cayenne Turbo, the centre piece of the air guide is a single-part.

| 2. | The bottom air guide -arrows- does not have to be removed. Remove retaining bracket (front apron engine guard) and move the lock carrier into the service position → 503809 Loosening and fastening lock carrier (service position) - section on "Loosening". |
| --- | --- |

| 3. |  |
| --- | --- |

| 4. | Use an appropriate collecting container for any Pentosin which runs out. Release hoses from radiator for power steering and close with suitable plugs. Remove the mount of the radiator for power steering -arrow- , only screwed on the left-hand side. |
| --- | --- |

Mount for the servo oil cooler

| 5. | Unclip the radiator for power steering from the mount -arrow- . |
| --- | --- |

| 6. | Bracket |
| --- | --- |

| 7. | Pull the -retaining frame for radiator- carefully to the front. |
| --- | --- |

| 8. | Carefully push the radiator module back. Pull the -cooler for power steering system- out between the radiator module and the retaining frame for the radiator. |
| --- | --- |

top of page

- Installing radiator for power steering

- Installing radiator for power steering

| 1. | Push the -cooler for power steering system- in between the radiator module and the retaining frame for the radiator. |
| --- | --- |

| 2. | Retract the radiator for power steering into the mount -arrow- . |
| --- | --- |

| 3. | Clip the radiator for power steering into the bracket -arrow- . Ensure correct seating. |
| --- | --- |

| 4. |  |
| --- | --- |

| 5. | Fit fastening screw -arrow- , connect hoses for power steering radiator and fit securing clamps. The hoses must not be twisted in the process. |
| --- | --- |

| 6. | Together with bottom air guide, move bumper out of service position and into normal position → 503819 Removing and installing lock carrier and fit retaining bracket (front apron engine guard). |
| --- | --- |

| 7. | Install top air guide -arrows- and lateral air guides. |
| --- | --- |

top of page

- Subsequent work

- Subsequent work

| 1. | Fit front apron → 631519 Removing and installing front apron - section on "Installing" . |
| --- | --- |

| 2. | Mount engine guard → 108019 Removing and installing engine guard - section on "Installing" |
| --- | --- |

| 3. | Fill Pentosin and bleed steering → Test and assembly work on power steering. |
| --- | --- |

| 4. | Lower the vehicle → Lifting the vehicle [9PAAD1 9PAAD7 9PAAE1 9PAAE7 9PAAF1]→ Lifting the vehicle [9PAAJ1]. |
| --- | --- |

9PAAD1, 9PAAD7, 9PAAE1, 9PAAE7, 9PAAF1, 9PAAJ1

as of MY 2003
Country C00, C02, C05, C07, C08, C09, C10, C11, C12, C13, C14, C15, C16, C18, C19, C20, C21, C22, C23, C24, C25, C26, C27, C28, C32, C33, C34, C35, C36, C37, C38, C39, C45, C46, C98, C99